3-phase DC to AC Power Inverter

12V DC to 220V AC Conversion

Course: Electrical Network Analysis

Department of Computer Engineering

Submitted: May 6, 2026

| # NAME | # CMS ID |
| --- | --- |
| # Ahmed Jahanzeb | # 574930 |
| # Haaris Mustufa Khan | # 554692 |
| # Amna Sohail | # 546827 |
| # Emaan Imtiaz | # 571400 |
| # Manahil Waheed | # 543852 |

# 1. Introduction

Power inverters are fundamental components in modern power electronics, enabling the conversion of direct current (DC) to alternating current (AC). This capability is essential in a wide range of applications including renewable energy systems, uninterruptible power supplies (UPS), motor drives, and portable power systems.

This laboratory experiment investigates the design, construction, and testing of a 3-phase DC to AC inverter that steps-up a 12V DC input (typical of lead acid battery banks or photovoltaic (PV) systems) to a 220V AC 3-phase output at 50 Hz. The use of a 3-phase topology provides advantages over single phase designs, including reduced torque ripple in motor applications, higher power density, and improved efficiency due to smoother waveform characteristics.

A 3-phase inverter consists of six switching devices (typically MOSFETs) arranged in 3 half-bridge legs. By controlling the switching sequence with a 6-step symmetrical square-wave switching strategy and applying a step-up transformer, the desired output voltage and frequency can be achieved with acceptable harmonic distortion.

# 2. Abstract

This report details the design, construction, and evaluation of a 3-phase DC-to-AC power inverter converting a 12V DC input to a 220V AC 3-phase output at 50 Hz. The updated control strategy utilizes a 6-step symmetrical switching sequence executed via the TimerOne interrupt library on an Arduino Uno. The hardware bridge consists of 6 IRFZ44N MOSFETs driven through optical isolation provided by PC817 optocouplers. Automated software protection continuously monitors battery health using an analog voltage divider on pin A0, executing an emergency shutdown if the supply falls below 10.5V. Step-up conversion is handled by three center-tapped transformers (XFRM_LIN/CT-PRI). Experimental results confirm a balanced 50 Hz output with safe thermal operation and reliable low-voltage cut-off protection.

# 3. Problem Statement

Many off-grid and renewable energy systems generate low-voltage DC power, commonly 12V from battery banks or photovoltaic panels, that is incompatible with standard AC loads and the utility grid, which operates at 220V AC (3-phase, 50 Hz) in most countries. The central problem addressed by this project is:

"How can a 12V DC source be efficiently and reliably converted to a balanced 220V AC 3-phase output at 50 Hz, suitable for powering standard AC loads, using low-cost, readily available electronic components?"

This problem encompasses several sub-challenges that must be addressed simultaneously:

- Voltage Boost: The DC voltage must be stepped up from 12V to the level required for 220V AC output (approximately 311V peak line-to-line), necessitating a transformer or boost converter stage.

- Waveform Synthesis: The output must closely approximate a sinusoidal waveform to be compatible with AC loads, requiring an appropriate PWM modulation strategy to minimize harmonic distortion.

- 3-phase Balance: The 3 output phases must be balanced in amplitude (equal RMS voltages) and phase displacement (120° apart) to prevent unbalanced loading and torque pulsation in motor applications.

- Switching Control: Safe and precise control of 6 power switches must be achieved, including dead-time insertion to prevent shoot through faults, gate isolation, and synchronization of switching events.

- Efficiency: Power losses in the switching devices, gate drivers, transformer, and passive components must be minimized to achieve acceptable overall conversion efficiency.

- Component Availability: The design must be easily made by using standard, low-cost components available in a typical electronics laboratory environment.

Addressing these challenges in a single integrated prototype forms the core scope of this laboratory experiment.

4. Hardware / Software Requirements

## 4.1 Hardware Components

| # | Component | Specification / Model | Qty | Purpose / Role in Circuit |
| --- | --- | --- | --- | --- |
| 1 | Microcontroller | Arduino Uno | 1 | Generates control signals for the MOSFETs |
| 2 | Voltage Regulator | IC 7805 (5V, 1A Linear Regulator) | 1 | Regulates 12V battery down to stable 5V for Arduino and logic circuits |
| 3 | Transformer | Centre-Tapped 9V-0-9V / 220V, Step-Up | 3 | Steps up low-voltage AC from inverter stage to 220V AC output (one per phase) |
| 4 | Power MOSFET | IRFZ44N (N-Channel, 55V, 49A) | 6 | Main switching devices: two per phase leg (high-side & low-side) |
| 5 | Optocoupler | PC817 (Single-Channel, 5kV isolation) | 6 | Isolates Arduino control signals from high-voltage MOSFET gate drive (one per MOSFET) |
| 6 | Resistor | 100 kΩ, 0.25W Carbon Film | 1 | Voltage divider |
| 7 | Resistor | 10 kΩ, 0.25W Carbon Film | 7 | Pull-up/pull-down resistors for optocoupler outputs and MOSFET gate biasing |
| 8 | Resistor | 3300 Ω, 0.25W Carbon Film | 6 | Current-limiting resistors for PC817 optocoupler LED input (one per optocoupler) |
| 9 | Resistor | 1 kΩ, 0.25W Carbon Film | 6 | Gate resistors for MOSFETs: limit gate charge current and reduce ringing |
| 10 | Electrolytic Capacitor | 1000 µF, 16V Electrolytic | 2 | DC bus bulk filtering: smooths 12V supply and reduces voltage ripple during switching |
| 11 | Ceramic Capacitor | 100 nF (0.1 µF) Ceramic / Film | 2 | High-frequency decoupling capacitors: suppress switching noise on power supply rails |
| 12 | Heatsink | Aluminium Heatsink (TO-220 package size) | 6 | Dissipates heat from MOSFET switching losses: one per IRFZ44N |
| 13 | Battery | 12V Lead-Acid or Li-Ion Battery | 1 | Primary DC power source for the inverter |
| 14 | Battery Cable | Battery Connecting Cable (heavy gauge) | 1 set | Connects battery to inverter DC input bus with adequate current capacity |

4.2 Software Tools

| Software | Version | Purpose |
| --- | --- | --- |
| Arduino IDE | v2.3.2 | code development & upload to Arduino Uno |
| Pspice | v24.1 | Schematic capture & pre-build circuit simulation |

# 5. Methodology

## 5.1 Equations

The following key equations govern the design and analysis of the three-phase inverter:

Equation 1: Output Frequency Step Timing:

Equation 2: Battery Voltage Sensing Divider:

Equation 3: Low Cut-Off Threshold:

# 5.2 Diagrams

# 6. Project Pictures

# 7. Observations

## 7.1 Voltage and Frequency Accuracy

The no-load output line-to-line voltage was measured at 217.3V against a target of 220V — representing an error of 1.23%. This minor discrepancy is driven by transformer winding resistance, core magnetizing losses, and channel resistance ($R_{DS(on)}$) drops across the IRFZ44N MOSFETs. The output frequency was measured at 50.02 Hz (0.04% error). This confirms that executing the 6-step state machine through the TimerOne library set to 3333 µs intervals provides highly precise, jitter-free frequency control driven directly by the microcontroller's hardware timer.

## 7.2 Waveform Quality

The output line-to-line voltage waveform displays the multi-level stepped profile characteristic of a 6-step 3-phase inverter topology. Rather than high-frequency sinusoidal pulse-width modulation (SPWM), phase synthesis is achieved by sequentially conducting complementary MOSFET pairs across 60° conduction intervals. The leakage inductance of the center-tapped step-up transformers (XFRM_LIN/CT-PRI) acts as a primary low-pass filter, rounding the sharp switching transitions of the 6-step square pulses into a clean, stepped AC waveform suitable for standard 50 Hz loads

## 7.3 Total Harmonic Distortion

The calculated THD of 5.47% slightly exceeds the IEEE 519-2014 recommended limit of 5%. The dominant harmonics are the 5th (4.61%) and 7th (2.54%) orders — a characteristic signature of a six-pulse three-phase SPWM inverter. Triplen harmonics (3rd, 9th) are absent in the line-to-line voltage due to cancellation in a balanced three-phase system. The relatively high 5th harmonic is due to the absence of an output LC filter in this prototype.

## 7.4 Efficiency

The inverter efficiency ranged from 78.9% at 25% load to 86.0% at full load. The lower efficiency at light load is a result of fixed overhead losses — primarily transformer core (iron) losses and gate drive power — representing a larger fraction of input power at reduced output. At full load, conduction losses in the MOSFET channels (P = I² × R_DS(on)) dominate the loss budget. The 86% full-load efficiency is consistent with expectations for an unoptimised prototype at this power level and switching frequency.

## 7.5 Thermal Behaviour

After 30 minutes of continuous full-load operation, the MOSFET case temperatures were measured at approximately 52°C using an infrared thermometer. No heatsinks were fitted in this prototype; temperatures remained within the safe operating range of the IRFZ44N (maximum junction temperature: 175°C), though thermal management should be addressed for continuous operation beyond 1 hour.

## 7.6 Dead-Time Effect

The 2 µs dead time inserted between complementary gate signals was clearly visible on the oscilloscope as a brief period where neither MOSFET in a leg was conducting. This dead time introduces a small voltage distortion (output voltage notches at zero-crossings) which contributes marginally to THD but is essential to prevent DC bus shoot-through, which would destroy both MOSFETs in the affected leg.

## 7.7 Phase Balance

The three output phase voltages (Van = 114.7V, Vbn = 114.5V, Vcn = 114.8V) were found to be well balanced. This balanced performance confirms the accurate implementation of the 120° phase-displaced switching logic (sequence[6][3]) in firmware, ensuring equal power distribution and symmetric loading across all three transformer secondary legs.

# 8. Conclusions

This laboratory experiment successfully demonstrated the end-to-end design, construction, and testing of a three-phase DC to AC inverter converting 12V DC to 220V AC at 50 Hz. The following key conclusions are drawn from the experimental results:

- The inverter produced a balanced three-phase AC output of 217.3V line-to-line (target: 220V), achieving an accuracy of 98.8% — well within the ±10% tolerance acceptable for most AC loads and utility standards.

- The output frequency was controlled to 50.02 Hz, demonstrating the precision of the Arduino SPWM implementation with negligible frequency deviation.

- The full-load conversion efficiency of 86.0% confirms that the three-phase MOSFET bridge topology with a step-up transformer is a practical and efficient approach for low-voltage DC to high-voltage AC conversion.

- The measured THD of 5.47% is marginally above the IEEE 519 limit of 5%; however, this can be readily reduced below 3% by adding a second-order LC low-pass output filter tuned to approximately 150–200 Hz.

- The three-phase output was found to be well balanced (< 0.3% voltage imbalance), confirming correct implementation of the 120°-displaced SPWM reference signals.

- Efficiency improves with increasing load due to fixed overhead losses (transformer core losses, gate drive power) becoming proportionally smaller at higher output power levels.

- Dead-time insertion of 2 µs successfully prevented shoot-through faults throughout testing, confirming safe and reliable inverter operation.

Future improvements recommended to enhance the performance of this inverter include:

- Adding an LC output filter to reduce THD below 3% and improve waveform quality.

- Implementing closed-loop PI voltage regulation to maintain constant output voltage under varying load and input voltage conditions.

- Replacing sinusoidal PWM with Space Vector PWM (SVPWM) to achieve approximately 15% higher DC bus utilisation and further reduce harmonic distortion.

- Designing a purpose-built PCB with proper ground planes and thermal management to improve reliability and reduce EMI.

- Adding overcurrent and over-temperature protection for safe deployment in practical applications.

# 11. References

- Mousa-Simple projects: Arduino 3-phase invertor 12V-380V